STELLA-1.1  Manual

STEPS:

1. Power the STELLA on

Press and release the power button. The display will show a welcome screen while the system is booting up. When the device is ready to use, the instrument table will appear.

2. Choose your data collection sampling

Tap the yellow circle on the touchscreen to enter single-sample mode.

3. Position the STELLA above the target.

Position the STELLA horizontally above the target so that the sensors are held a few inches away from the target.

4. Take a sample measurement.

- Calibrate using the white reference panel.

- Record the distance, calibration batch number, and material name.

- Take two measurements of your material and write them down.

- Take a second calibration (this can serve as the first for the next material).

- Always calibrate before and after each material.

5. Analyze the Data.

- Remove the SD card and insert it into your computer using the SD-to-USB-C adapter.

- Go to the provided website

- Upload the .csv file, enter distance and batch numbers.

- Record reflectance values and plot them to create a spectral signature graph.

- Repeat for each material.

STELLA-Q2  Manual

STEPS:

1. Power the STELLA on

Press and release the power button. The display will show a welcome screen while the system boots up. When the device is ready, the instrument table will appear.

2. Choose your data collection sampling

Select your desired sampling mode by pressing the button.

- Using the built-in light:

  - If you want to use the built-in light to measure a sample placed directly against the device, hold the button for 5 seconds to turn the light on. When you see the light turn on, release the button.

  - To record data, press the button again — when the button lights up, wait 3 seconds while the measurement is taken. Once the light turns off, you can record another sample or perform a calibration.

3. Position the STELLA above the target.

In this activity, we use the built-in LED light. Place the STELLA directly against the material with the light on.

4. Take a sample measurement.

Press the button to take a measurement. The light will flash while data is being recorded.

- As with the STELLA-1.1, take two calibration measurements (before and after) and two sample measurements for each material.

- Record the distance, batch numbers, and material name in your notes.

- Note: STELLA-Q2 automatically records three data points per measurement for better accuracy, but you still record two data points. Also, the values are not visible on the screen after each reading.