## Part 1: Software Installation and Testing

### Step 1: Connect Components for Initial Testing

Connect the sensors in this order using the small cables:

- Microprocessor → Time Clock – insert CR1220 battery

- Time Clock → Button

- Button → Display

Large cables:

- Display → Magnetometer

- Magnetometer → Particulates

Small cables:

- Particulates → CO2

- CO2 → Barometer

### Step 2: Install CircuitPython

- Hold down the BOOT button on the RP2040 microprocessor while connecting it to your computer via USB-C

- The device will appear as an RP1-RP2 drive

- Drag and drop the UF2 file onto the drive (replace any existing files)

- The device will reboot and appear as CircuitPy drive

### Step 3: Install Software Libraries

- Once the CircuitPy drive appears, drag and drop the code.py and lib folder onto the drive

- Go to your test codes folder and drag the code.py file for the clock onto the drive

### Step 4: Set the Real-Time Clock

- Navigate to time.is/UTC to check the current UTC time

- Open your code editor and open the serial monitor

- Set the time in UTC format

  - Tip: Add 5-10 seconds to the current time to account for saving delays

- Save the time settings and verify it saved correctly

- Check the time multiple times to ensure accuracy

### Step 5: Install Main Operating Code

- From your code and libraries folder, drag code.py into the CircuitPy drive

- Make sure to overwrite any other code that was previously there

- The STELLA-AM should boot up and the screen should display data

- Test to ensure everything lights up properly

## Part 2: Preparing the 3D Printed Housing

### Step 6: Clean the 3D Prints

Before assembly, you need to remove support material from the printed parts:

Top Housing:

- Remove 4 support structures

- Clean out supports from underneath the clips

Bottom Housing:

- Remove 5 support structures

- Pay special attention to supports under the clips

- Remove the large support in the middle section

Tip: Use an exacto knife to ensure clean prints, especially in tight areas and retaining bars

## Part 3: Installing Components in Housing

### Step 7: Install the Time Clock (Most Difficult Component)

- Locate the retainer marked for the clock (retainers have letters marking their positions)

- Clean up the retaining bar with an exacto knife if needed for proper fit

- Slide the time clock into its retainer

- Make sure it fits snugly but don't push too hard

- The bar should slide in easily once properly cleaned

- Trim any rough edges if necessary

### Step 8: Install Microprocessor

- Flip the housing with the long side down

- Insert the microprocessor into its designated retainer

- Ensure it sits nice and snug (no need to push down too hard)

### Step 9: Install Particulate Sensor

- Slide the particulate sensor under its retainer

- Snap it into place

- The particulate sensor helps hold the time clock bar in position

### Step 10: Install CO2 Sensor

- Curl the wire slightly for better wire management

- Insert the CO2 sensor into its retainer

- Tuck it right in next to the barometer position

### Step 11: Install Barometer

- Place the barometer in its retainer next to the CO2 sensor

- They should sit side by side

- Get the retaining clip and install it from side to side

- The clip holds both the CO2 and barometer sensors in place

- Give it a little nudge to ensure both are held securely

### Step 12: Install Magnetometer

- The magnetometer has its own dedicated clip

- Pop the magnetometer into its retainer with the clip

### Step 13: Install Button

- Make sure the button orientation is correct before installing

- Place the button in its retainer

- Clip the button down with its retaining clip

## Part 4: Final Assembly

### Step 14: Wire Management and Screen Installation

Note: Wire length can be an issue. You may need to rearrange wires for proper fit.

- If needed, pull out the particulate sensor temporarily

- Rearrange wires to get more length for the screen connection

- Reinstall the particulate sensor and button

- Wrap and organize the wires neatly

- Ensure the screen has adequate wire length and clearance

### Step 15: Install the Display Screen

- Insert the screen into the top lid/housing

- Slide the retaining door down to secure it

- Verify everything fits properly

- Tuck the wires back into the housing

- Check that all components are properly positioned

### Step 16: Close the Housing

- Verify all wires are properly tucked in

- Snap the top and bottom housings together

- Ensure all clips engage properly

### Step 17: Install the Translucent Window

- Place the 3D printed translucent lamp window into position

- This allows you to see indicator lights while protecting the components

## Part 5: Power Up and Test

### Step 18: Final Boot and Testing

- Connect the USB-C power cable

- The instrument should boot up automatically

- Verify the screen displays data properly

- Check that all sensors are reporting values

- Confirm data is being recorded to the micro-SD card

Congratulations! You've successfully built a STELLA-AM!

## Troubleshooting Tips

- Wire length issues: Rearrange wire connections or carefully extend wire routing

- Tight 3D print fits: Use an exacto knife to clean up edges and support material

- Screen clearance: Adjust wire positioning and ensure proper seating in the top housing

- Component retention: Ensure all clips are properly engaged and retainers are clean

## Important Notes

- The instrument is modular - you can use any subset of sensors if working with a limited budget

- All components are commercially available

- Complete component cost: approximately $180 (2025 prices)

- Power: USB-C (no internal battery except coin cell for clock)

- Data format: CSV files with time/date stamps on micro-SD card

- Housing dimensions: approximately 6" x 3" x 1"

- Total weight: 115g (4 oz)